This operation instruction is for the LCD keypad used on multi-line AMD signs, one or more AMD signs grouped together to form a "Multi-line AMD Display".  The AMD LED Display is controlled by an external Keypad connected to the sign with a RJ-11 cable (not a telephone cable).

The initial LCD display screen after power up is the UPLOAD Message:

"UPLOAD ID xx"  where xx is the ID of your sign.  This is normally "01" (zero one).  Ensure the ID is correct for your sign.  You may change the ID by pressing the "+" or "-" keys.  Press enter to confirm the ID and upload the current sign configuration and prices.  Note:  Some keypads will upload the sign automatically.

The initial LCD display screen after power up and upload is the EDIT Message menu:

The LCD would display:

"  LINEx: 88.88  ", where x is the line (display unit) number and could be from 1 through 6, and where 88.88 is the current price on the sign (e.g. 21.75).  If the format / price shown is not of the correct format you must format the sign (see section B).

- Edit Message menu:

- To enter digit value: Press numerical keys ('O' through '9') to enter the desired number, or press 'F1' key to select blank value (space) which would blank out the digit on the sign.

- To select a different display line (or display unit): Press '+' key to move down one line (increase) and '-' to move up one line (decrease).

- After all prices have been keyed in, press the 'ENTER' key to send all prices to the sign.

- To go to Dimming Selection menu: Press 'Dimmer Set' key.

- Digit Format selection menu:

This menu is used to select different digit formats. E.g. 88.99, 1.88 or 8888, etc., for the LED display.

To go to Digit Format selection menu: Press 'Swap Set' key.

The LCD would display:

"FORMATx: 88.88 ", where x is the line (display unit) number and could be from 1 through 6, and 88.88 is the format of the sign.  This should match the display pattern of the actual sign.  For gas prices, it should be 8.889.

- To select digit format: Press 'Review' key to cycle through all possible formats.  The 'Review' key will cycle that digit through 1,8,9,., and blank.  Use the arrow keys to move to the next or previous digit.

- To select display line (or display unit): Press '+' key to move down one line (or unit) and '-' to move up one line (or unit).  Press 'ENTER' key to send out the selection to the unit.

- Ensure that you only have a format for the number of lines on your sign and the remaining lines are blank.  e.g. 3 product sign should have a format for line 1, 2, 3 and all blank for line 4, 5, 6.

- To go to Edit message menu: Press 'EDIT' key.

- To go Dimming Selection menu: Press 'Dimmer Set' key.

- Dimming Selection Menu:

This menu is used to select proper brightness level and/or control method (auto or manual brightness control) of display units.  The values of the brightness level are:

Setting	Power level

1		100%

2		60%

3		30%

4		15%

5		8%

The LCD would initially display:

"MODE: 1=A 2=M [1]", where 'A' means auto mode and 'M' means manual mode.

- To select auto or manual mode: Press '1' key to enter auto mode and press '2' key to enter manual mode.  You may also press '+' key or '-' key to select the number.

Auto mode:

The LCD would display:

"DM HIGH 1-5[x]", where x indicates the brightness level during daytime.  '1' is the brightest setting and '5' is the lowest.

- Press '+' key or '-' key to select different daytime brightness level.

- Press 'ENTER' key to set the level.  Then, the LCD would display:

"DM LOW 1-5 [x]", where x indicates the current brightness level during nighttime.

- Press '+' key or '-' key to select different night time

brightness level.

- Press 'ENTER' key to set the level.

Manual mode:

The LCD would display:

"DM SET 1-5[x]", where x indicates the current brightness level.

a.  Press '+' or '-' key to select desired brightness level.

- Press 'ENTER' key to set the level.

- To go to Edit message menu: Press 'EDIT' key.

- To go to Digit Format Selection menu: Press "Swap Set' key.